# Ex.No:17 PERFORMANCE EVALUATION OF DIGITAL COMMUNICATIONSYSTEMS USING EYE DIAGRAM ANALYSIS UNDER ADDITIVE WHITE GAUSSIAN NOISE (AWGN) USING MATLAB

Objective 1:

Code:

clc; clear; close all;

%% Parameters

N = 100;	% Number of bits

sps = 20;	% Samples per symbol

SNR = 10;	% Signal-to-Noise Ratio (dB)

%% Generate Random Binary Data data = randi([0 1], 1, N);

%% NRZ Encoding (0 -> 0, 1 -> 1)

tx_signal = repelem(data, sps);

%% Pass Signal Through AWGN Channel signal_power = mean(tx_signal.^2); noise_power = signal_power / (10^(SNR/10));

noise = sqrt(noise_power) * randn(size(tx_signal)); rx_signal = tx_signal + noise;

%% Time Axis

t = (0:length(tx_signal)-1)/sps;

%% Plot Results figure;

% Original Binary Data subplot(4,1,1);

stairs(0:N-1, data, 'LineWidth', 1.5); grid on;

title('Original Binary Data'); xlabel('Bit Number'); ylabel('Bit');

ylim([-0.2 1.2]);

% Transmitted NRZ Signal subplot(4,1,2);

plot(t, tx_signal, 'LineWidth', 1.5); grid on;

title('Transmitted NRZ Signal'); xlabel('Time (Bit Periods)'); ylabel('Amplitude');

ylim([-0.5 1.5]);

% Received Signal After AWGN subplot(4,1,3);

plot(t, rx_signal, 'r'); grid on;

title(['Received Signal with AWGN (SNR = ', num2str(SNR), ' dB)']); xlabel('Time (Bit Periods)');

ylabel('Amplitude');

% Eye Diagram (manual plotting; no eyediagram function required) subplot(4,1,4);

hold on;

for k = 1:2:(N-2)

idx = (k-1)*sps + 1;

segment = rx_signal(idx:idx+2*sps-1); time_segment = (0:2*sps-1)/sps; plot(time_segment, segment, 'b');

end

grid on;

title('Eye Diagram of Received Signal'); xlabel('Time (Bit Periods)'); ylabel('Amplitude');

xlim([0 2]); hold off;

Output: