# Ex.No:18 - MATLAB SIMULATION AND EVALUATION OF EYE DIAGRAM PARAMETERS UNDER INTER-SYMBOL INTERFERENCE IN DIGITAL COMMUNICATION SYSTEMS

## Objective 1:

Code:

clc; clear; close all;

%% Parameters

N = 100;	% Number of bits

sps = 20;	% Samples per symbol

%% Generate Random Binary Data data = randi([0 1], 1, N);

%% NRZ Encoding

tx = repelem(data, sps);

%% Time Axis

t = (0:length(tx)-1)/sps;

%% Introduce Inter-Symbol Interference (ISI)

% Simple Low-Pass FIR Channel h = [0.2 0.6 1 0.6 0.2];

% Received Signal with ISI rx = conv(tx, h, 'same');

%% Plot Results figure;

% Original Binary Data subplot(4,1,1);

stairs(0:N-1, data, 'LineWidth', 1.5); grid on;

title('Original Binary Data'); xlabel('Bit Number'); ylabel('Bit');

ylim([-0.2 1.2]);

% Transmitted NRZ Signal subplot(4,1,2);

plot(t, tx, 'LineWidth', 1.5); grid on;

title('Transmitted NRZ Signal'); xlabel('Time (Bit Periods)'); ylabel('Amplitude');

ylim([-0.5 1.5]);

% Received Signal with ISI subplot(4,1,3);

plot(t, rx, 'r', 'LineWidth', 1.2); grid on;

title('Received Signal with Inter-Symbol Interference (ISI)'); xlabel('Time (Bit Periods)');

ylabel('Amplitude');

% Eye Diagram under ISI subplot(4,1,4);

hold on;

for k = 1:2:(N-2)

idx = (k-1)*sps + 1;

segment = rx(idx:idx+2*sps-1); time_segment = (0:2*sps-1)/sps; plot(time_segment, segment, 'b');

end

grid on;

title('Eye Diagram under ISI'); xlabel('Time (Bit Periods)'); ylabel('Amplitude');

xlim([0 2]); hold off;

Output: