Ex.No:9: PERFORMANCE EVALUATION OF AM GENERATION AND DETECTION UNDER NOISY CHANNEL CONDITIONS USING MATLAB

Objective 1:

Code:
clc;

clear;

close all;

% Parameters

fs = 100000;

t = 0:1/fs:0.02;

fm = 500;

fc = 5000;

% Message Signal

m = cos(2*pi*fm*t);

% Carrier Signal

c = cos(2*pi*fc*t);

% AM Modulation

am = (1 + m).*c;

% Add AWGN Noise (10 dB SNR)

snr_db = 10;

signal_power = mean(am.^2);

noise_power = signal_power / (10^(snr_db/10));

noise = sqrt(noise_power) * randn(size(am));

noisy = am + noise;

% Envelope Detection

env = abs(hilbert(noisy));

% Recover Message Signal

recovered = env - mean(env);

% Display Results

figure;

subplot(5,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

grid on;

subplot(5,1,2);

plot(t,c);

title('Carrier Signal');

xlabel('Time (s)');

grid on;

subplot(5,1,3);

plot(t,am);

title('AM Signal');

xlabel('Time (s)');

grid on;

subplot(5,1,4);

plot(t,noisy);

title('AM Signal with AWGN Noise');

xlabel('Time (s)');

grid on;

subplot(5,1,5);

plot(t,recovered);

title('Recovered Message Signal');

xlabel('Time (s)');

grid on;

Output: