Ex.No:24: ATLAB SIMULATION OF PASSBAND TRANSMISSION WITH BER ANALYSIS UNDER MULTIPATH FADING CHANNELS

Objective 1:

Code:

clc;

clear;

close all;

%% Parameters

N = 1000;

SNR_dB = 10;

%% Generate Random Bits

bits = randi([0 1], N, 1);

%% BPSK Modulation

% 0 -> -1, 1 -> +1

tx = 2 * bits - 1;

%% Multipath Fading Channel

h = [1 0.5 0.3];

fade = filter(h, 1, tx);

%% Add AWGN (Without Communications Toolbox)

signalPower = mean(fade.^2);

SNR_linear = 10^(SNR_dB / 10);

noisePower = signalPower / SNR_linear;

noise = sqrt(noisePower) * randn(N, 1);

rx = fade + noise;

%% Detection

bits_rx = double(rx > 0);

%% BER Calculation

numErrors = sum(bits ~= bits_rx);

BER = numErrors / N;

%% Display Results

fprintf('BPSK Multipath Channel Simulation\n');

fprintf('SNR = %d dB\n', SNR_dB);

fprintf('Total Bits = %d\n', N);

fprintf('Number of Bit Errors = %d\n', numErrors);

fprintf('BER = %.6f\n', BER);

%% Plots

figure;

subplot(3,1,1);

plot(tx);

title('Transmitted BPSK Signal');

xlabel('Bit Index');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

plot(rx);

title('Received Signal (Multipath + AWGN)');

xlabel('Bit Index');

ylabel('Amplitude');

grid on;

subplot(3,1,3);

stem(1:50, bits(1:50), 'b', 'filled');

hold on;

stem(1:50, bits_rx(1:50), 'r');

title(['Original vs Detected Bits (BER = ' ...

num2str(BER, '%.4f') ')']);

xlabel('Bit Index');

ylabel('Bit Value');

legend('Original', 'Detected');

ylim([-0.2 1.2]);

grid on;

hold off;

Output: