Ex.No:25: SIMULATION OF CONVOLUTIONAL CODES AND ANALYSIS OF ENCODED OUTPUT BIT SEQUENCES USING MATLAB

Objective 1:

Code:

clc;

clear;

close all;

%% Input Bits

data = randi([0 1], 20, 1);

%% Convolutional Encoder

% Rate = 1/2

% Constraint Length = 3

% Generator Polynomials = [7 5] (octal)

m = 2;                      % Memory elements

state = zeros(m, 1);        % Initial encoder state

N = length(data);

encoded = zeros(2*N, 1);

for i = 1:N

% Current input and previous bits

u = data(i);

% Generator polynomial 7 (111 in binary)

out1 = mod(u + state(1) + state(2), 2);

% Generator polynomial 5 (101 in binary)

out2 = mod(u + state(2), 2);

% Store encoded output bits

encoded(2*i-1) = out1;

encoded(2*i) = out2;

% Update encoder state

state(2) = state(1);

state(1) = u;

end

%% Display Results

disp('Input Bits:');

disp(data');

disp('Encoded Bits:');

disp(encoded');

%% Plot Results

figure;

subplot(2,1,1);

stem(1:N, data, 'filled');

title('Input Bit Sequence');

xlabel('Bit Index');

ylabel('Bit Value');

ylim([-0.2 1.2]);

grid on;

subplot(2,1,2);

stem(1:length(encoded), encoded, 'filled');

title('Convolutionally Encoded Bit Sequence');

xlabel('Encoded Bit Index');

ylabel('Bit Value');

ylim([-0.2 1.2]);

grid on;

Output: